1. In orthographic projection, the exact form of the object is shown by various views of the object arranged in a particular order. a.; True. b.; False. ANSWER:; True.

2. The purpose of a chain line is to indicate the imaginary cut surface referred to by the cutting plane line. a.; True. b.; False. ANSWER:; False.

3. There is no limitation on the number of views that may be used to describe an object. a.; True. b.; False. ANSWER:; True.

4. The front view always shows the front of the object. a.; True. b.; False. ANSWER:; False.

5. All views have a particular position with respect to each other, and have either a horizontal or vertical alignment. a.; True. b.; False. ANSWER:; True.

6. ____ are made to describe the object in sufficient detail to permit fabrication. a.; Details; b.; Plans. c.; Drawings; d.; Graphs. ANSWER:; c.

7. ____ projection is the method employed to describe the object in sufficient detail to permit fabrication. a.; Sectional; b.; Series. c.; Drawing; d.; Orthographic. ANSWER:; d.

8. A broken line of medium thickness is known as a(n) ____ line. a.; hidden; b.; object. c.; center; d.; leader. ANSWER:; a.

9. A fine, broken line made up of a series of short and long dashes alternately spaced is known as a(n) ____ line. a.; hidden; b.; leader. c.; center; d.; object. ANSWER:; c.

10. ____ lines are fine lines that extend from the object with a slight break between. a.; Extension; b.; Object. c.; Hidden; d.; Dimension. ANSWER:; a.

11. A fine, straight line with an arrowhead or round solid dot at one end is known as a(n) ____ line. a.; extension; b.; leader. c.; hidden; d.; object. ANSWER:; b.

12. A heavy, broken line made up of a series of long and short dashes alternately spaced is known as a ____ line. a.; break; b.; section. c.; chain; d.; short. ANSWER:; c.

13. The ____ break line is a heavy, irregular line drawn freehand. a.; phantom; b.; cutting. c.; long; d.; short. ANSWER:; d.

14. The ____ break line is a ruled, light line with freehand zigzags. a.; short; b.; long. c.; cutting; d.; phantom. ANSWER:; b.

15. When three views are used to describe an object, the ____ view is placed directly above and in line with the front view. a.; top; b.; right. c.; bottom; d.; left. ANSWER:; a.

16. A(n) ____________________ line is a thick solid line. ANSWER:; object.

17. A series of lines – solid or solid and broken – arranged in specific patterns are known as ____________________ lines. ANSWER:; section.

18. A(n) ____________________ line is a light, broken line made up of a series of one long and two short dashes. ANSWER:; phantom.

19. When three views are used to describe an object, the ____________________ side view is placed to the right of and in line with the front view. ANSWER:; right.

20. The ____________________ view usually gives the best indication of the shape and detail of an object. ANSWER:; front.